Features

- Standard Frequency Range：3850～4550MHz@VT=0V～5V

- Output Power：≥8dBm @VCC=5V

- Ripple(BW≤200MHz)：±0.5dB

- Supply Voltage(VCC)：4.2V～6V

- Supply Current：16mA@VCC=5V

- Harmonic Suppression(2nd)：≤-25dBc

- Harmonic Suppression(3rd)：≤-25dBc

- No External Components Needed

- 50Ω Load Impedance

- 7mm×9mm×2mm SMT Package

Product Description

The YSGM404506 voltage-controlled oscillator (VCO) employs a highly stable oscillation circuit design ,delivering high output power and superior isolation. It is powered by a standard 5V supply, with compatibility across a 4.2 to 6V range.Featuring an extensive tuning voltage span of 0-5V, it facilitates broadband frequency output. The built-in filter and impedance matching circuit minimizes the need for external components while ensuring consistent output power stability. The oscillator's output is readily connectable to a 50Ω load. Encased in a compact 7mm×9mm×2mm package, the device is designed for ease of integration and space efficiency.

Pin Description

| Pin | Symbol | Function | Pin | Symbol | Function |
| --- | --- | --- | --- | --- | --- |
| 1 | RFOUT | RF output | 5 | VT | Tune voltage |
| 2 | GND | Ground | 6 | GND | Ground |
| 3 | GND | Ground | 7 | VCC | Supply voltage |
| 4 | GND | Ground | 8 | GND | Ground |

Absolute Maximum Ratings

| Parameter | Rating | Unit | Parameter | Rating | Unit |
| --- | --- | --- | --- | --- | --- |
| Tune Voltage | 0 ~ 5 | V | Storage Temperature | -40 ~ +150 | °C |
| Supply Voltage | 4.2~ 6 | V | Relative Humidity | ＜80% | RH |
| Operating Ambient Temperature | -40 ~ +85 | °C | Atmospheric Pressure | 85 ~ 106 | KPa |

| Specification |  |  |  | Unit | Condition |
| --- | --- | --- | --- | --- | --- |
|  | Min. | Typ. | Max. |  |  |
| Lower Frequency | 3800 | 3850 | 3900 | MHz | VT=0V |
| Upper Frequency | 4500 | 4550 | 4600 | MHz | VT=5V |
| Power Output |  | +8 |  | dBm | VCC=5V |
| Tune Voltage | 0 |  | 5 | V |  |
| Supply Current |  | 16 |  | mA | Open&Load |
| Leakage Current(VT) |  |  | 12 | μA | VT=5V |
| Pushing（VCC） |  | 10 |  | MHz/V | VT=5V |
| Pulling（VSWR） |  | 10 |  | MHz pp | VSWR=3:1 |
| Drift Rate |  | 0.4 | 0.6 | MHz/°C |  |
| Load Impedance |  | 50 |  | Ω |  |
| Harmonic(2nd) |  |  | -25 | dBc |  |
| Harmonic(3rd) |  |  | -25 | dBc |  |

Electrical Specifications（T=+25°C，VCC=5V）

| Typical Performance |  |
| --- | --- |
| Frequency vs. Tuning Voltage vs. Operating Voltage | Frequency vs. Tuning Voltage vs. Temperature |
| Power vs. Tuning Voltage vs. Operating Voltage |  |

Outline Drawing

TOP VIEW                                 SIDE VIEW

BOTTOM VIEW

UNIT: mm

TOLERANCE: ±0.1mm

PCB Land Pattern

UNIT: mm